Introduction:

The Automatic Tap Using an IR Sensor is an embedded optoelectronic system designed to automate fluid distribution. Traditional plumbing reliance on human physical force to open and close gate valves or ball valves is inefficient. This system replaces mechanical friction with a touchless electronic control loop. The device operates on the physical principle of diffuse light reflection within the non-visible electromagnetic spectrum. The IR transmitter emits a continuous beam of light at a wavelength of approximately 940 nanometers. When no object is present, this light scatters into the surrounding environment, leaving the receiver inactive.

When a user places their hands in the sink basin, the skin reflects a portion of the infrared photons back to the photodiode. This light exposure drops the photodiode's internal resistance, generating a voltage swing. The control circuit processes this change, triggering the relay module to complete the power circuit for the solenoid valve. The valve's internal electromagnetic coil pulls open its sealing plunger, letting water flow. Once the hands are removed, the light reflection stops, the control signal drops, and an internal spring forces the valve shut, instantly stopping the water flow.

Problem Statement:

Modern fluid distribution infrastructure faces significant challenges regarding conservation, hygiene, and accessibility. Manual water faucets contribute to resource depletion and public health issues in several key areas.

Standard manual faucets are highly inefficient because water flow is binary (fully open or fully closed) and dependent on human intervention. Significant fluid wastage occurs during intermediate activities, such as applying soap, scrubbing skin, or brushing teeth, when water flows directly down the drain without being used.

In public restrooms, users frequently fail to shut off manual taps completely, or leave them running due to negligence. This continuous flow wastes approximately 6 to 12 litres of treated water per minute per faucet. Over time, turning mechanical handles too hard wears down rubber gaskets and washers, causing slow, chronic leaks that waste hundreds of litres daily per facility.

In high-traffic environments—such as public transit hubs, medical institutions, schools, and commercial complexes—faucet handles act as primary vectors for indirect contact pathogen transmission (fomites). When an individual with contaminated hands opens a manual tap, they transfer pathogens (including E. coli, influenza strains, and various noroviruses) onto the handle. After washing with soap, the user must touch the exact same contaminated handle to shut off the water. This re-contaminates their clean hands, undermining the hygienic purpose of washing. The moist environment around tap handles also encourages biofilm growth, turning them into long-term breeding grounds for bacteria.

In high-traffic environments—such as public transit hubs, medical institutions, schools, and commercial complexes—faucet handles act as primary vectors for indirect contact pathogen transmission (fomites). When an individual with contaminated hands opens a manual tap, they transfer pathogens (including E. coli, influenza strains, and various noroviruses) onto the handle. After washing with soap, the user must touch the exact same contaminated handle to shut off the water. This re-contaminates their clean hands, undermining the hygienic purpose of washing. The moist environment around tap handles also encourages biofilm growth, turning them into long-term breeding grounds for bacteria.

Objectives:

Fluid Conservation and Optimization

The primary objective is to optimize water usage by aligning water delivery exactly with the duration of active washing. The project aims to design an automated valve control system that eliminates human delay and forgetfulness. By using precise sensor positioning and minimizing relay lag, the system can reduce average fluid wastage by up to 60–70% compared to standard manual faucets.

Contactless Sanitary Control

The project aims to improve hygiene by replacing mechanical handles with a touchless interface. Utilizing an active infrared detection arc eliminates the need for physical contact during water activation and deactivation. This breaks the cross-contamination cycle, improves public health protection, and lowers infection rates in shared environments.

Economically Viable Engineering

A core objective of this mini-project is to design a high-efficiency automation solution using accessible, low-cost components. Rather than relying on expensive industrial plumbing systems, the design uses consumer-grade electronics, such as the LM393 operational amplifier comparator, standard infrared diodes, and budget-friendly 5V/12V electromechanical actuators. This keeps the design affordable and highly scalable for residential, educational, and small business settings.

Electrical Isolation and Safety Compliance

Because the system controls conductive fluids using electric current, a key engineering objective is to implement a robust power isolation framework. The system must completely isolate the user-facing low-power detection logic (operating at 5V DC, <100mA) from the higher-power actuator switching network (operating at 12V DC or higher). This is achieved by using optocouplers and electromechanical relay modules, ensuring galvanic isolation to prevent electrical shock or system short-circuits.